# Summary table of article content on feature extraction

| No. | Title | DOI | Criteria | Summarize |
| --- | --- | --- | --- | --- |
| 1 | Feature extraction and selection for objective gait analysis and fall risk assessment by accelerometry | https://doi.org/10.1186/1475-925X-10-1 | Q2 | The article extracts 67 features from 20 elderly individuals (70 years and older) to assess their potential risk of falling. The data is collected from a network of 10 sensors placed across the body while performing a 25-meter straight-line walking test. The Forward Wrapper Selection algorithm is used to choose the most suitable feature pairs, and several classification models are used to evaluate the effectiveness of each feature set. |
| 2 | Acceleration Feature Extraction of Human Body Based on Wearable Devices | https://doi.org/10.3390/en14040924 | Q1 | This study focuses on preprocessing the dataset using the Kalman filter algorithm, further optimized by the Kalman-Genetic Algorithm (KGA) to significantly reduce computational complexity. Subsequently, feature extraction is performed using statistical features (Standard Deviation, SD) and waveform-based structural features (Interval of Peaks, IoP; Difference between Adjacent Peaks and Troughs, DAPT). A machine learning model from prior literature was employed to validate the effectiveness of the extracted features, with a comparative analysis against existing feature extraction methods. |
| 3 | Data Feature Extraction Method of Wearable Sensor Based on Convolutional Neural Network | https://doi.org/10.1155/2022/1580134 | Q2 | This article presents a method for extracting features from five datasets using a CNN model. The five different datasets were filtered using the Extended Kalman algorithm, and the data was then segmented into individual samples, which served as the input to the CNN model. Subsequently, the first output consists of the extracted features, which were also used for classifying the test dataset within the CNN model to show the final result. (The dificult reference because of the dataset and the model). |
| 4 | ECG signal feature extraction trends in methods and applications | https://doi.org/10.1186/s12938-023-01075-1 | Q2 | This article extracts four feature domains from an ECG dataset: Time-domain, Frequency(-Time) domain, Joint Time-Frequency domain, and Decomposition domain. Furthermore, within each domain section, the article presents a table comparing the advantages and disadvantages of each feature extraction algorithm, following a detailed explanation of the underlying theory and associated formulas. In conclusion, the paper is structured as a review that does not present the exact results of each algorithm or any empirical testing, but instead provides a deeper understanding of the features within each domain. |
| 5 | Trends in biomedical signal feature extraction | https://doi.org/10.1016/j.bspc.2018.02.008 | Q1 | The article focuses on analyzing trends in feature extraction from biomedical signals in detail, with a particular emphasis on the classification of feature extraction methods based on four distinct domains. Furthermore, within each domain, the paper provides a comprehensive review of the most prominent algorithms, accompanied by a detailed table outlining their advantages, disadvantages, and practical applications. In addition, experiments were conducted to evaluate the effectiveness of the proposed approaches. Finally, the paper presents a comparative analysis across domains in the conclusion section. |
| 6 | A Comparison of Feature Extraction Methods for the Classification of Dynamic Activities From Accelerometer Data | 10.1109/TBME.2008.2006190 | Q1 | This study focuses on comparing the effectiveness of different feature extraction methods, namely wavelet features, time-domain features, and frequency-domain features, in classifying dynamic activities. Two datasets were used for evaluation, collected from accelerometer sensors mounted at three body locations: the waist, thighs, and ankles. In conclusion, the study employs the k-Nearest Neighbors (kNN) algorithm to assess the efficiency of the extracted features. The results indicate that time-domain and frequency-domain features are generally more effective than wavelet features in most cases involving a single sensor, with the most pronounced improvement observed for the ankle-mounted sensor. |
| 7 | A Study on the Essential and Parkinson’s Arm Tremor Classification | https://doi.org/10.3390/signals2020016 | Q2 | This study first provides a clear theoretical overview of tremor-related diseases before presenting the feature extraction methods and applied classification models. The dataset was collected using four acceleration sensors mounted on the thumb, index finger, back of the hand, and wrist while volunteers performed four distinct tests. Twelve features were extracted from the time domain, specifically statistical features, and 21 classification algorithms across five categories (Trees, Discriminant Analysis, SVM, KNN, and Ensemble Learning) were employed to classify the volunteers' conditions. In conclusion, SVM algorithms demonstrated the highest performance in training models, with the Coarse Gaussian SVM achieving the best classification results in the motion-with-object test |

Bảng so sánh tổng quan về trích xuất đặc trưng

| Domain | Description | Advantages | Disadvantages |
| --- | --- | --- | --- |
| Time Domain | Phân tích tín hiệu theo thời gian, sử dụng các đặc trưng như giá trị trung bình, phương sai, mô hình autoregressive (AR), Linear Predictive Coding (LPC). | - Đơn giản, trực quan - Giữ nguyên thông tin thời gian - Thích hợp cho dữ liệu có độ nhiễu thấp | - Không hiệu quả với tín hiệu không dừng, phi tuyến - Khó phát hiện thông tin tần số hoặc pha của tín hiệu​ |
| Frequency Domain | Chuyển tín hiệu sang miền tần số bằng Fourier Transform, Discrete Fourier Transform (DFT), Hilbert Transform, Discrete Cosine Transform (DCT). | - Bộc lộ các đặc trưng phổ tần số rõ ràng - Hữu ích cho tín hiệu có tính tuần hoàn - Thích hợp cho nén dữ liệu và xử lý ảnh | - Mất thông tin thời gian của tín hiệu gốc - Không phù hợp với tín hiệu không dừng |
| Joint Time-Frequency Domain | Kết hợp cả hai miền thời gian và tần số bằng Short-Time Fourier Transform (STFT), Continuous Wavelet Transform (CWT), Wavelet Packet Transform (WPT). | - Phân tích tốt tín hiệu không dừng, phi tuyến - Kết hợp ưu điểm của hai miền - Có khả năng loại bỏ nhiễu tốt hơn | - Tốn nhiều tài nguyên tính toán - Cần chọn cửa sổ và thông số thích hợp để tối ưu hóa kết quả​ |
| Decomposition & Sparse Domain | Phân tích tín hiệu thành các thành phần nhỏ hơn, sử dụng Empirical Mode Decomposition (EMD), Singular Value Decomposition (SVD), Compressive Sensing. | - Giảm thiểu dư thừa thông tin, chỉ giữ lại đặc trưng quan trọng - Tốt cho xử lý tín hiệu phi tuyến, không dừng - Phù hợp với dữ liệu lớn và machine learning | - Phức tạp, yêu cầu nhiều tính toán - Cần chọn thuật toán phù hợp để tránh mất thông tin quan trọng​ |